User Manual

Audio RMS Display: an Arduino sound-level meter with a 10-segment LED bar and a browser monitor

1. Overview

The file firmware.ino contains the complete program and is already flashed to the Arduino. The meter works on its own: plug in power and the LED bar shows the ambient sound level. The browser monitor is optional. It adds numeric readouts and lets you change the sensitivity.

2. Getting started

- Connect the Arduino to the computer with a USB cable. The LED bar starts working right away.

- Open rms-meter-monitor.html in Google Chrome or Microsoft Edge. Firefox and Safari do not support the Web Serial interface, so the Connect button will not work there.

- Click Connect, pick the Arduino from the port list, and confirm. The status changes to “Connected · 115200” and live values appear within about a second.

Note: connecting resets the board, so the sensitivity goes back to its default of 120. The monitor picks up the correct values automatically from the incoming data.

3. Operation

- Clap, speak, shout, or play music. The bar rises with the loudness of the sound (the RMS level). More lit LEDs means a louder signal.

- A single marker LED above the bar shows the recent peak. It holds for two seconds and then falls back slowly.

- The monitor mirrors the physical bar and shows the RMS, peak, DC bias, raw peak-to-peak, and the current sensitivity.

4. Adjusting the sensitivity

Use More sensitive / Less sensitive in the monitor, or send '+' or '-' from any serial terminal at 115200 baud. If the bar sits at full scale all the time, click Less sensitive. If it barely reacts, click More sensitive. The monitor shows the current value, and the device confirms every change.

5. Health indicator

- signal alive: the microphone delivers a normal signal.

- quiet: very little signal. The room is silent or the microphone gain is set too low.

- mic dead? A0 flat: the input does not move at all. Check the KY-038 wiring and its power supply.

6. Folder contents

The submission folder contains everything needed to review the project and to debug it:

- Presentation: the final slide deck.

- Circuit diagram: KiCad source files for the full schematic.

- Contribution statement: a one-page summary of each student's work.

- Codes: all source files for the project.

  - firmware/ firmware.ino, the complete program that runs on the Arduino.

  - backup/ holds two development sketches: a KY-038 calibration file and an LED bar graph test.

  - rms-meter-monitor.html is the browser serial monitor. It mirrors the physical LED bar and shows the live readings over Web Serial.